DIMENSIONS:		SPEEDS:

LENGTH	87‘ 10‘‘	MMO	 .85 MACH

WING SPAN	69‘ 06‘‘	VMO	335 KIAS

HEIGHT	20‘ 08‘‘	TURBULENT AIR.....What Comes First	.280 / 0.75

HORIZONTAL STAB. WIDTH	20‘ 04‘‘	VFE  8 / 20 	215 KIAS

MAIN TO MAIN GEAR	10‘ 04‘‘	VFE 30	185 KIAS

NOSE TO MAIN GEAR	36‘ 04‘‘	VFE 45	170 KIAS

GROUND TO BASE OF CARGO BAY	05‘ 01‘‘

GROUND TURNING RADIUS	min 75‘	VLO EXT.	250 KIAS

MAX LOAD ON PAX DOOR	1.000 lbs	VLO RET.	200 KIAS

MAX NUMBER OF PAX ON STAIRWAY	PAX 4	VLE	250 KIAS

MAX AFT CARGO	3500 lbs

AFT CARGO FLOOR LOADING	75lbs / in2	VMAX WIPER  OPS	220 KIAS

OVERHEAD BIN CAPACITY	45 lbs	VTIRE	182 KIAS

GREEN LINE	STALL WARN1.06 VS	1.27    Vso

WEIGHTS:

MAX RAMP	53.250	51.250 lbs	24.154 kg	VMAX ADG DEPLOYED  (330 / 12min  - 335 / 4min)	250 KIAS

MAX T/O	53.000	51.000 lbs	24.040 kg	MB	 .75 MACH

MAX LANDING	47.000 lbs	21.319 kg	VB	280 KIAS

MAX ZERO FUEL	44.000 lbs	19.958 kg	VMCG 20° FLAPS	84 KIAS

EMPTY WEIGHT	30.292 lbs	13.740 kg	V1MCG 	90 KIAS

MAX FUEL	14.305 lbs	6.488 kg

MAX PAYLOAD	13.708 lbs	6.218 kg	NORMAL CRUISE ECONO	 .74 MACH

MIN FLIGHT	30.000 lbs	13.608 kg	NORMAL CRUISE FAST	 .77 MACH

MIN OPERATIONS	25.480 lbs	11.558 kg	HIGH SPEED, 95% MTOW, FL 350	 .81 MACH

USABLE FUEL:	PRESSURE	GRAVITY	OPERATION PARAMETERS:

LEFT MAIN	700  gls	4.760 lbs	4.488 lbs	MAX OPERATING ALT	41.000’

RIGHT MAIN	700  gls	4.760 lbs	4.488 lbs	MAX T/O & LANDING	10.000’

CENTER	735  gls	4.998 lbs	4.930 lbs	MAX TEMP T/ O & LANDING	ISA +35° C

TOTAL	2.135  gls	14.518 lbs	13.906 lbs	MIN TEMP   T/ O & LANDING	-40° C

USABLE FUEL:	PRESSURE	GRAVITY	RUNWAY SLOPE	+/- 2%

LEFT MAIN	2.650 ltr	2.156 kg	2.036 kg	COMPONENT	T- WIND	10 KIAS

RIGHT MAIN	2.650 ltr	2.156 kg	2.036 kg	25° RUDDER	X-WIND	27 KIAS

CENTER	2.782 ltr	2.267 kg	2.237 kg	33° RUDDER	X-WIND	24 KIAS

TOTAL	8.082 ltr	6.585 kg	6.309 kg	WET RNWY	X-WIND	15 KIAS

ICY   RNWY	X-WIND	15 KIAS

CAT IIIA (50’ 700RVR) 	X-WIND	13 KIAS

MAX FUEL IMBALANCE:	800 lbs	363 kg

CAUTION MESSAGE	800 lbs	181 kg	RUNWAY CONTAMINATION:

0.5’’ STANDING WATER	1’’ WET SNOW

MIN FOR G/A PER WING	450 lbs	204 kg	0.5’’ SLUSH  (0.88’ LNDG)	3’’ DRY SNOW

AUX TO MAIN TRANSFER	94%	97%

MIN FUEL TEMP FOR T/O	41°F	5°C	MAX  REVERSE BELOW 60 KIAS	30%N1

FUEL CONSUMPTION APU:	120 lbs	55 kg	LOAD FACTORS:

1ST HOUR (both engines)	3.500 lbs	1.587 kg	FLAPS UP	-1g TO 2.5g

2nd  HOUR (both engines)	2.400 lbs	1.088 kg	FLAPS DOWN	0g TO 2.0g

THEREAFTER (M .74)	2.200 lbs	998 kg	ICING THRESHOLD	  10°C & VISIBLE MOIST

ENDURANCE APRX. 7 HRS	1.620 NM	3.000 km	ICING WING + COWL 5°C & VISIBLE MOIST

Fuel system computer initiates auto crossflow upon detecting a fuel imbalance of 200 lbs. (91 kg) between mains, until 50 lbs. (23 kg)

ITT:		FLIGHT CONTROLS:

MAX NO APR 	FIRST 2 MIN.   900° C	FLAPS MAX ALT	15.000’

ELEC. CONTR. N1 79%	NEXT 3 MIN.   884° C	FLAPS EN ROUTE USE > 4°	PROHIB.

MAX WITH APR	FIRST 2 MIN.   928° C

APR LOGIC    <67% - >79%	NEXT 3 MIN.   900° C	SPOILERS MIN SPEED	VREF +17

APR  ICONS OF N1 INDICATORS 5MIN AFTER TAKEOFF		FAA SPOILERS MIN ALTITUDE	800’

MAX CONTINUOUS THRUST	874° C	JAA SPOILERS MIN ALTITUDE	300’

OIL TEMP:		SPOILER & SPOILERON DEPLOY LOGIC:

MIN START	- 40° C	ARMED + L & R THRUST LEVER	IDLE

MAX CONTINUOUS	155° C	  PLUS TWO OF THE FOLLOWING CONDITIONS

MAX  (15 min)	163° C	A: L OR R MLG	WOW

B: RADAR ALT	BELOW  5’ feet

OIL PRESSURE:		C: BOTH WHEEL SPEEDS	ABOVE  16 kts

MIN IDLE	25 psi

MIN AT T/O THRUST	45 psi	SPOILER & SPOILERON AUTO RETRACT:

MAX CONTINUOUS	115 psi	L & R THRUST LEVER	< MIN FOR T/O

MAX AFTER COLD START	156 psi	INBD & OUTBD WHEEL SPEED	>45for10sec

AIRPLANE ON THE GROUND FOR	>40sec

130 psi at idle, for max 10min, must remain at idle until oil pressure within normal limits 115 psi.

MANUAL RETRACT	DISARM SWITCH

MAX CONSUMPTION 6.4 ounces / hr	0.05 gph

MAX REFIL WITHOUT MOTORING	2 qts	FLIGHT SPOILERS DETENTS	8 avail

FOUR MARKED POSITIONS

MAX DURATION WITHOUT ENGINE OIL	16 hrs	 (¼  - 8°)    ( ½  - 19°)    ( ¾ - 33°)   (MAX - 50°)

N1:

MAX T/O & CONTINUOUS	98.6 %

NORMAL T/O	96.2 %

N2:

START........< 30.000’  300KTS  90° & 12%    25s /  30s	20 %

IDLE	56 % TO  68 %

T/O WITH APR MAX	99.4 %

CONTINUOUS MAX	99.2 %

T/ O NORMAL	98.3 %

IGNITION & STARTER OFF	55 %

ICING CONDITIONS MIN	64 %

WING ANTI-ICE ON & OFF	99.3 %

STARTER LIMITS:	ITT BELOW 120°C	APU 30 KVA:

1st  &  2nd   START	1 min ON	10 sec OFF	EGT MAX CONT.	743 °C

3rd &  4th    START	1 min ON	5 min OFF	EGT MAX START	974 °C

Dry Motor -not during start-	90sec	RPM MAX	107 %

AIR STARTS:	ITT BELOW 90°C

BELOW    15.000’	300 KIAS, 9% - 55% N2	OAT MIN START	- 40 °C

OPS ALT MAX	37.000’

15.000’ OR BELOW	ENGINE X-BLEED > 60 psi	STARTING ALT MAX .............(7%)	30.000’

13.000’ OR BELOW	START FROM APU	BLEED USAGE ALT MAX	15.000’

DOOR ALT v.s. SPEED	13.000’ – 230/260 KTS

ADG MAX SPEED       160 kts	250 - 330	12 min	DOOR OPEN MAX WITH APU OFF	300 KTS

NO LIMIT BELOW 250      34.4KVA	330 - 335	4 min	10%- IGN ON         50%- MOTOR OFF        95%   IGN OFF

AUTOMATIC FLIGHT CONTROLS:		ELECTRICAL:

SPEED MODE ON APROACH	PROHIB.	IDG < 35.000’		30 KVA

APRCH MODE ON VOR APR	PROHIB.	IDG > 35.000’		25 KVA

ENROUTE MIN ALTITUDE	600’		GARET    GTCP - 36 - 150RJ

NON-PRECISION MIN ALT	400’	APU - 37.000’	97%-102%	30 KVA

PRESITION AROACH FAA	80’	ADG > 160 KTS	< 90KTS OFF  LINE	15 KVA

ALL GENS 12.00 RPM  400 HZ   115V & 200V  3 PHASE    5 TRU’s @ 100 A

AUTOPILOT DICONNECT:

SWICH BAR ON FCP	LOWERED

AP / SP DISC SWITCH ON YOKE	PRESSED	APU	BATT 24 V	43 AH

TO / GA BUTTON	PRESSED	CHRGR	TPNG 10 A	40 AH

YAW DAMP DISC SWITCH	PRESSED	MAIN	BATT 24 V	17 AH

STABILISER TRIM	OPERATED	CHRGR	TPNG   5 A	20 AH

FCC ONE OF TWO	FAIL	TPNG	OVER CHARGE	115 %

BOTH YAW DAMPENERS	FAIL	GND OPS 	DC ONLY	5 min

RIGHT GENERATOR	ON / OFF	AIR OPS 	BATT ONLY	30 min

AIR CONDITIONING & PRESSURIZATION:		HYDRAULICS:	SKYDROL

MAX NEG DIF	- 0.5 psi	MAX PRESS DISCHARGE	3.750 psi

T/O & LANDING MAX DIF	0.1 psi	HIGH PRESS	3.200 psi

NORMAL DIF	8.4 psi	NORMAL PRESS	+/- 100	3000 psi

WARNING DIF	8.5 psi	LOW PRESS	1.800 psi

MAX DIFF RELIEF	8.7 psi	3 ACCUMULATORS	1.500 psi

CABIN ALT CAUTION	8.500’	SYSTEM 1	EDP 1     &     ACMP 1b

CABIN ALT WARNING	NO SMKG	10.000’	ELEVATOR	R & L

CPAM PAX OXY DROP	+/- 500’	14.000’	FLIGHT SPOILERS	R & L

EMERG DEPRESS SEL	-250 / +500’	14.250’	OUTBRD GND SPOILERS	R & L

NO MANUAL OPS BELOW	-1.500’ 	AILERON	L

SPOILERON	L

SINGLE PACK OPS	25.000’	RUDDER	SYST COOLING BY AIR&FAN

PACK OVER HEAT	RESET @ 85°C	> 88 °C

PACK OVER PRESS	RESET @ 51psi	> 53 psi	SYSTEM 2	EDP 2     &    ACMP 2b

ELEVATOR	L & R

CARGO BAY OVERHEAT	95°F / 35°C	FLIGHT SPOILERS	L & R

RESET POSSIBLE AT	85°F / 29°C 	INBRD GND SPOILERS	L & R

AILERON	R

BLEED AIR:	ICE T/O & LNDG PACKS on APU	SPOILERON	R

DOWNLOCK ASSIST	MLG

7TH	STAGE	ENGINE OIL SEAL PRESS	BRAKES	OUTBRD

ENGINE SUMP VENTING	RUDDER	SYST COOLING BY AIR&FAN

10TH	STAGE   ENVIROMENT		SYSTEM 3	ACMP 3a  &  ACMP 3b

- PRESSURISATION		ELEVATOR	L & R

- AIR CONDITIONING		SPOILERON	L & R

- AVIONICS COOLING		AILERON	L & R

- ENGINE STARTING		LANDING GEAR	MAIN

LANDING GEAR	NOSE

GND EXT AIR    PRESS / VOL	        1.5 psi  /  70 lbs min

STEAR BY WIRE		7° / 70° / 75’

14TH	STAGE   ANTI-ICING		BRAKES	INBOARD

- WING ANTI ICE		RUDDER	SYST COOLING BY  FUEL SYSTEM

- COWL ANTI ICE		ADG RUNS ACMP 3b/ B-PMPS AUTO FLAPS NOT ZERO

- THRUST REVERS

APU STARTING BATT.  POWER:	(15 s Ext. Power)	LANDING GEAR:

1ST  START	30 SEC ON	NONE	NOSE WHEEL PRESS	146 psi

2ND START	30 SEC ON	20 MIN	MAIN WHEEL PRESS	168 psi

3RD START	30 SEC ON	NONE	500’ AGL NO GEAR	“TO LOW GEAR”

4TH START	30 SEC ON	40 MIN	MLG OLEO EXT	MIN 1.25’’	MAX3.75’’

ENGINE	GEN. ELETR. CF 34 - 3A1(3B1)	GEAR HORN TL IDLE	<185 KIAS

3A1	FLAT RATED ISA	+  7°C	30° FLAPS & GEAR UP	<163 KIAS

3B2	FLAT RATED ISA	+15°C	45° FLAPS & TL IDLE	CANT’T MUTE

T/O  THRUST WITH APR	9.220  lbs	TAXI LIGHTS	 NOT MOVING	10  min

APR +2% N1  APX 500 lbs	85% BYPASS	+ 491  lbs	LANDING LIGHTS ON	< 10.000’

FAN SPEED 	 100%   N1	RATIO 6.2 - 1	7.400 rpm	TAXI L-ENG ONLY	HYD 2B ON

CORE SPEED	 100%   N2		17.280 rpm	TAXI IN SLUSH	FLAPS  UP

VIBRATION:		BRAKES:

VIB > 3.6 mils	WARNING MSG	NORMAL PRESS	1.000  -  3..200 psi

VIB > 2.7 mils	AMBER VIB IN N2	ANTI SKID	>35 KTS,  WOW,  5 sec DELAY

VIB > 1.7 mils	RED VIB MSG	ANTI SKID	STOPS WORKING	10 KTS

VIB SENSOR RIGHT SIDE OF ENGINE CORE, RIGHT	2 BRAKE ACCUMULATORS	750 psi

SIDE SLIGHTLY OVER SENSITIVE LIVE WITH IT		BRAKING APLICATIONS      ANTI SKID OFF	6

MIN TURN AROUND TIME	15 min	BTMS  1-5Grn  6-11Wht  12-20Red	unit 35°C

DURING SINGLE ENGINE OPS, APU BLEED AIR IS	BRAKE WARE PINS CHECKED WITH HYDR.

NOT TO BE USED FOR ENG START USE X-BLEED		SEYTEMS 2 & 3 PRESSURISED BRAKES SET