| CLOCK SETTING |  |  |
| --- | --- | --- |
| Select GPS time on FMS CDU, Add 3 minutes to time, Use computed time to set clock |  |  |
| Set selector switch to SET | GMT minutes flashes | Use CHR to set GMT minutes |
| Push ET switch | GMT hours flashes | Use CHR to set GMT hour |
| Push ET switch | LOC minutes flashes | Use CHR to set LOC minutes |
| Push ET switch | LOC hours flashes | Use CHR to set LOC hour |
| Push ET switch | DY (Day) flashes | Use CHR to set DY (Day) |
| Push ET switch | MO (Month) flashes | Use CHR to set MO (Month) |
| Push ET switch | YEAR flashes | Use CHR to set YEAR |
| When GPS clock rolls to time used for setting immediately: |  |  |
| Set selector switch to GMT | Verify GMT time |  |
| Set selector switch to LOC | Verify LOC time |  |
| Set selector switch to DATE | Verify Date |  |
| Set selector switch to GMT | Verify clock operation |  |
| Verify FMS times on CDU (IDENT page) |  |  |

| STAB TRIM SETTINGS |  |  |  |  |
| --- | --- | --- | --- | --- |
| FUEL LOAD | BOW | GTOW | % MAC | TRIM SETTING |
| 10,000 | 51,200 | 61,200 | 32.1 | 6.4 |
| 16,000 | 51,200 | 67,200 | 29.8 | 7.4 |
| 22,000 | 51,200 | 73,200 | 28.8 | 7.9 |
| 28,000 | 51,200 | 79,200 | 29.6 | 7.7 |
| 35,000 | 51,200 | 86,200 | 29.4 | 7.8 |
| 40,000 | 51,200 | 91,200 | 28.1 | 8.4 |
| 43,400 | 51,200 | 94,600 | 26.8 | 8.7 |
| Takeoff Stab Settings are approximate |  |  |  |  |

| POWER-UP FAULT CAS TROUBLESHOOTING |  |  |  |
| --- | --- | --- | --- |
| CAS MESSAGE | EMS PAGE | C/B | ACTION |
| AV BATT FAIL | CCBP C-5 | AV BATT CHRG | RESET if Amber box is around Avionics battery on DC synoptic. This message can occur if BATT is turned "OFF" before RPM is "0" and APU door is shut. |
| APU BATT FAIL | ACPC | APU BATT CHRG | RESET |
| APU BATT FAIL | ELEC PANEL | APU GEN | OFF / RESET |
| SLAT FAIL |  |  | None required, failure should clear on movement of SLAT/FLAP Lever |
| SLAT / FLAP BIT | CNTL | SLAT/FLAP RESET | RESET: Message should clear when IAS > 60 kts. |
| SLAT / FLAP HALFSPD | CNTL | SLAT/FLAP RESET | @ POWER UP ONLY |
| STALL WARN ADVANCE | CNTL | STALL WARN ADVANCE | RESET |
| SHAKER 1 FAIL | FLT CNTRL | SPC A | RESET |
| SHAKER 2 FAIL | FLT CNTRL | SPC B | RESET |
| AFCS 1 FAIL | AFCS | IAC 1 | RESET |
| AFCS 2 FAIL | AFCS | IAC 2 | RESET |
| CB TRIP | STATUS | TRU BAY FAN CTL | ACKNOWLEDGE |
| FDR FAIL |  |  | None required. Message will clear with engine start |
| L/R FADEC FAULT |  |  | None required. Message will clear 20 minutes after engine start. |
| PITCH FEEL FAULT FLT SPOILERS FAIL FLT SPOILERS FAULT RUD LIMITER FAIL RUD LIMITER FAULT RUD AUTHORITY | FLT CNTR | FLT CTL 1 CH A FLT CTL 1 CH B FLT CTL 2 CH A FLT CTL 2 CH B | If any of these messages remain after powering up all hydraulic pumps, pull all 4 C/B's, then Reset in any order. Should take about 1 minute to clear. If unsuccessful, try engine start and pressurizing all hydraulic systems, then try C/B reset. |
| BRAKE FAULT | LDG GEAR | BRAKE CTL CH A BRAKE CTL CH B | Pull both C/B's then reset. (NORM BRAKE FAIL message will appear). This message will appear if A/C is powered-up while towing. (APU started while AC in motion). |
| TAT HT 1-2-3 FAIL PITOT 1-2-3 HT FAIL ICE DETECTOR FAULT | ICE | HBMU 1 HMBU 2 | Pull out both C/B's and reset. |
| L/R PACK FAULT | AIR COND | L/R PACK CTRL CH A L/R PACK CTRL CH B | First select TRIM AIR "OFF" and applicable PACK "OFF". If message does not clear, pull applicable C/B and Reset. Then reselect PACK and TRIM AIR. It is not unusual to see this message while on APU bleed in Hot/High elevations. If this is the case, switching to engine bleed will clear the message eventually. |
| TCAS FAIL | NAV | TCAS | RESET |
| PREDICTIVE RAIM UNAVAILABLE | NAV | GPS 1 & GPS 2 | RESET |

| EICAS MESSAGE TROUBLESHOOTING |  |  |  |  |
| --- | --- | --- | --- | --- |
| CAS MESSAGE | EMS PAGE | C/B |  | ACTION |
| ACARS DMU FAIL (FMS) | COMM | DATALINK |  | RESET |
| TRU 1 (2) FAIL |  | CCBP-A2 CCBP-C2 |  | RESET CHECK DC ELECTRIC PAGE, IF TRU IS IN "AMBER BOX" (GREEN BOX IS NORMAL) AND READINGS ARE NORMAL. SUSPECT FAN FAILURE. |
| AMBER HP VALVE (ON SYNOPTIC PAGE) |  |  |  | RECYCLE BLEED SWITCH |
| NO LIGHTS AFT CABIN (UNKNOWN REASON) |  |  |  | IF JUST OUT OF MAINTENANCE HOLD OXY. MASK TEST SWITCH TO RESET FOR 2 CHIMES |
| L(R) FADEC N¹ CTL |  |  |  | ENGINE MUST BE SHUT DOWN AND A/C DE-POWERED TO RESET. |
| IRU's WILL NOT ALIGN |  |  |  | RAD ALT MUST BE AT "O" FEET. CHECK RAMP LEVEL, GRATES UNDER ANTENNA ETC. : REPOSITION A/C IF NECESSARY |
| STALL WARN ADVANCE WITH ICE MESSAGE | CNTL | STALL WARN ADVANCE |  | RESET CAN OCCUR IF ANTI ICE WAS NOT IN "AUTO", AND ICE WAS DETECTED BEFORE ANTI-ICE WAS TURNED "ON". RESET STALL WARNING ADVANCE AFTER ANTI-ICE IS WORKING. |
| REFUEL PROBLEMS |  |  | ACTION |  |
| FULL FUEL LOAD |  |  | 1. WINGS SHOULD BE LEVEL, TO PREVENT REFUEL INHIBIT AT 400 POUND INBALANCE (1100 W/SB) 2. AUTO REFUEL UNTILL FMQGC ANTICIPATES HIGH LEVEL SENSOR SHUTOFF. 3. CONTINUE TO FUEL MANUALLY TO BYPASS HIGH LEVEL ANTICIPATORS, FUEL VALVES WILL SHUTOFF WHEN TANKS ARE FULL. SHORT VERSION: On arrival fuel wings to 24,000 using auto, on departure day top off tanks using manual (all tanks simultaneously). |  |
| VENTING |  |  | SCAVENGE PUMPS RETURN EXCESS FUEL FROM THE OVERFLOW VENT LINES BACK TO THE WING TANKS. SCAVENGE PUMPS ONLY OPERATE WHEN THERE PRIMARY PUMPS ARE RUNNUNG. TO PREVENT VENTING: IF PARKED FOR EXTENDED PERIODS ON APU POWER, OPEN CROSSFLOW AND RUN L/R PRIMARY PUMPS FOR 5-10 MINUTES AFTER FILLING WINGS. |  |
| REFUEL INHIBIT |  |  | VERIFY ONLY ONE PANEL (INTERIOR OR EXTERIOR) IS POWERED. IN AUTO: 1. VERIFY WING INBALANCE < 400 # 2. VERIFY ALL MANUAL SWITCHES "OFF" 3. RESET START, RE-INITIALIZE QUANTITY OR SWITCH TO MANUAL FUELING. IN MANUAL: 1. VERIFY ALL SWITCHES IN MANUAL. 2. VERIFY DESIRED TANK SWITCHES ARE OPEN. 3. REINITIALIZE: A) ALL SWITCHES "OFF" B) POWER "OFF" THEN "ON". C) OPEN DESIRED TANKS. D) REPEAT PROCESS IF NECESSARY. ENSURE THAT PMAT IS NOT HOOKED UP ! (INHIBITS REFUELING) |  |